|  |  |  | Aircraft; Final Approach; Short Final; Touchdown. A; 1.4 x VSO; 1.4 x VSO; VREF. B-D; VREF + 20 kts; VREF + 10 kts; VREF. Aircraft A: VREF < 91 kts; Aircraft B, C or D: VREF > 91 kts (For Detailed Aircraft Landing Categories see Table in "Procedure Turn" below) Copyright 1998 Wagner B. Beskow W.B.Beskow@massey.ac.nz FREEWARE DESIGNED FOR FLIGHT SIMULATION | Handy Sheet 3.0 |
| --- | --- | --- | --- | --- |
| Glide Slope 50 ft (AGL) Over RWY Threshold L A N D I N G |  |  |  |  |
| NM | Feet |  |  |  |
| 15 | 4780 |  |  |  |
| 14 | 4460 |  |  |  |
| 13 | 4140 |  |  |  |
| 12 | 3820 |  |  |  |
| 11 | 3500 |  |  |  |
| 10 Main V Speeds (kts IAS) V1 takeoff decision speed VR rotate speed V2 takeoff safety speed VS stall speed (clean) VSO stall speed (landing configuration) VREF landing reference speed (1.3 x VSO) VMC minimum control speed Rules of Thumb (a) TOD (Top of Descent) = 3 x (Current Altitude - Airport Elevation) (e.g. 3 x 39,000ft - 1,500ft elev = 113nm from airport). Use (b) to set the appropriate vertical speed based on TAS. Add 10% for unexpected tail wind. (b) Descent Rate (for 3o glide slope) = 5 x TAS (e.g. 5 x 400KTAS = 1200ft/min). (c) Descent Altitude Check = glide slope angle x 100 x Distance from RWY in nm (e.g. 3 x 100ft x 10nm = 3,000ft). For 4 o, 400 x Distance; for 5 o, 500 x D etc. (d) TAS = [(IAS x 2%) x (ALT/1,000ft)] + IAS. So, if IAS=300 and ALT=20,000: 300 x 0.02 x 20 + 300 = 420 kts TAS. (e) Bank Angle for Std Turns (i.e. Props 3o/s or 360o in 2 min; Jets 1.5o/s or 360o in 4 min). Props: BA = (TAS / 10) +7; Jets: BA = (TAS / 10) x 0.75. Airliners: plan BA to be max. 25o for passenger's comfort. (d) Rollout Angle = BA / 2. (e.g. current HDG = 090; desired HDG = 270; BA = 30; RA = 30 / 2 = 15; so start RA when HDG 285 (i.e. 15o before reaching HDG 270 ). | 3180 |  |  |  |
| 9 | 2870 |  |  |  |
| 8 | 2550 |  |  |  |
| 7 | 2230 |  |  |  |
| 6 | 1910 |  |  |  |
| 5 | 1590 |  |  |  |
| 4 | 1270 |  |  |  |
| 3 | 960 |  |  |  |
| GMT = Z = UTC 2 Reciprocal RWY Help 36  18 01  19 02  20 03  21 04  22 05  23 06  24 07  25 08  26 09  27 10  28 11  29 12  30 13  31 14  32 15  33 16  34 17  35 18  36 N (+) W (-) + E (+) S (-) | 640 |  |  |  |
| 1 | 320 |  |  |  |
| 0.5 | 160 |  |  |  |
| Thr GS = TAS - Headwind Comp. or GS = TAS + Tailwind Comp. | 50 |  |  |  |
| TD | 0 |  |  |  |

Standard 3o Glide Slope

Distance from RWY Threshold

Set altimeter to 1013hPa/29.92inHg passing through transition level (USA 18,000ft; NZ 11,000up/13,000dwn)

Feet Above Ground Level

MET reports' WINDS are in TRUE North;  ATC reported WINDS are MAG North

Don't debug anything on final,  GO AROUND !

oC           oF

-60     -76

-50     -58

-30     -22

-25     -13

-20       -4

-15        5

-10      14

- 5       23

0       32

5       41

10       50

15       59

20       68

25       77

30       86

35       95

40     104

50     122

A small correction early is better than a big correction late

HEADWIND/TAILWIND AND CROSSWIND COMPONENTS

Triangles: Upper-left shows headwind or tailwind component depending on whether the wind is coming from ahead or behind.  Lower-right shows left or right crosswind component also depending on wind direction.

CONVERSIONS

1 ft = 0.305 m

1 kts = 1 NM / hour

1 NM = 1.1516 SM = 1.853 km

1 SM = 0.8683 NM = 1.609 km

Air Pressure

inHg  		hPa

33.86

(follow multiply, back divide)

A   ALPHA		. —

B   BRAVO		— . . .

C   CHARLIE	— . —  .

D   DELTA		— . .

E   ECHO		.

F   FOXTROT	. . —  .

G   GOLF		—  — .

H   HOTEL		. . . .

I    INDIA		. .

J    JULIET		. — — —

K   KILO		—  . —

L    LIMA		. —  .

M   MIKE		—  —

N    NOVEMBER	— .

O    OSCAR		—  —  —

P     PAPA		. —  —  .

Q    QUEBEC	— —  . —

R    ROMEO	. — .

S    SIERRA		. . .

T    TANGO	—

U   UNIFORM	. . —

V   VICTOR	. . . —

W  WISKEY	. — —

X   XRAY		— . . —

Y   YANKEE	— . — —

Z   ZULU		— — . .